COMPOSITES					Ref: 001.1.16

PRODUCT DATA SHEET

HAND RAIL 1551-36-25

Description:

This Glass Reinforced Plastic handrail system is designed for surface mounting only. The profiles are produced with the pultrusion process in ISO polyester resin using a grey pigment as standard. Other pigments available on request.

Typical Uses:

- Urban-Rural Bus Stops.

- Pedestrian Road Crossing.

- School Safety Railing.

- Crowd Control Rail.

- Pedestrian Bridge Safety Rail.

- Shopping Malls.

- Hospital Access Ways.

- Commercial Property Boundary Railing.

- Government Offices.

- Military Installations

- Airport Perimeter.

- Hotels and Restaurants.

Features & Benefits.

- A steel replacement product.

- Equal lateral load capacity to steel.

- Manufactured from glass reinforced plastic.

- 100% corrosion resistant.

- Maintenance Free.

- UV Resistant.

- Special U shaped hand rail for safety.

- High abrasive resistance.

- High chemical resistance.

- Light weight.

- Assembled using stainless steel rivets.

Recommended Application.

- It is recommended to use this Hand Rail as follows.

- Occasional Access and General Duty.

- Lateral Load capacity – 0.36kn/m.

- Maximum stanchion span – 1600mm c/c.

- See separate Data Sheet for Installation.

Page1.

HAND RAIL 1551-36-25

Performance Data.

Fire Retardant.				Class 1 Rating per ASTM E – 84.

Abrasive Resistance.				High.

Cleaning.					Normal detergent

Chemical Resistance.	Will resist attack from most acids, alkalis oils, caustic solutions, sea water, grease etc.

Lateral Load Factor.				0.36kn/m.

Force.						360kg. acceleration / meter / second2.

Max. Stanchion Span.			1600mm.

Vertical Knee Rail Free Space.		250mm.

Optional Kick Plate.				150mmx5mmx3mm.

Permitted Usage.				Occasional Access & General Duty.

Manufacturers Standards:

- BE EN ISO 14122-3:2001.

- BS 6399-1:1996.

- BS 4592-0:2006.

- Products manufactured to client’s own design.

Hand Rail 1551-36-25 Product manufactured to client’s specification.

Tel:	+234 80 601 80786

Email:	sales@cic-wa.com					Page.2.