PROCEDURE FOR CALCULATING TAPE QUANTITIES.

Pipeline Criteria.

- Pipe Diameter.

- Pipe Length.

- Welded or Flanged.

- Operating Temperature: Min/Max.

- Pipe Condition: Old/New.

- Backfill Selected.

Pipeline Calculation Formula:

- Area of Tape Required:

= 3.142 x D x L x W.

( W – w )

Where:

- D	=	Pipe 0.D. 		(m)

- L	=	Pipe Length.	(m)

- W	=	Tape Width.	(mm)

- w	=	Overlap.		(mm)

Example:

300mm. NB. (324mm OD) pipe. 10m length wrapped with 150mm wide tape with 55% (82mm) overlap.

- Area of Tape Required:

= 3.142 x 0.324 x 10 x .150

0.068				(0.150mm less 55%)

= 22.45m2 of tape material

One roll of 150mm x 20m tape:		 =3m2.

Number of rolls:	22.45			7.48 rolls

3

P.1. 001